📡 ESP32-S High-Performance RF Monitor

A lightweight, high-speed RF monitoring solution for Amateur Radio. This firmware is optimized for the ESP32-S and ST7789 (240x320) TFT, providing real-time metrics with zero network overhead.

🛠 Hardware Integration: Pin-by-Pin Mapping

| Component | Pin Name | ESP32-S GPIO | Physical Pin Location |
| --- | --- | --- | --- |
| ST7789 TFT | VCC | 3.3V | Pin 1 (Top Left) |
| ST7789 TFT | GND | GND | Any GND pin |
| ST7789 TFT | SCL (Clock) | GPIO 18 | D18 |
| ST7789 TFT | SDA (MOSI) | GPIO 23 | D23 |
| ST7789 TFT | RES (Reset) | GPIO 4 | D4 |
| ST7789 TFT | DC (Command) | GPIO 2 | D2 |
| ST7789 TFT | CS (Chip Sel) | GPIO 5 | D5 |
| ST7789 TFT | BLK (Light) | GPIO 21 | D21 |
| RF BRIDGE | FWD Voltage | GPIO 34 | Analog Input (ADC1_6) |
| RF BRIDGE | REV Voltage | GPIO 35 | Analog Input (ADC1_7) |
| RF BRIDGE | Common GND | GND | Connect to ESP32 GND |

🧪 Technical Foundations

1. Power Calculation

The firmware utilizes the Power-to-Voltage relationship of a rectified directional coupler. Since $P = \frac{V^2}{R}$, and the bridge provides a linear voltage relative to the RF amplitude, we apply a scaling constant (K-Factor):

$$P = K \cdot (V_{fwd} + V_{diode})^2$$

- K-Factor (12.40): Maps the bridge’s DC output to Watts based on a $20\text{W}$ at $1.27\text{V}$ reference.

- Diode Compensation (0.17V): Corrects for the forward voltage drop of Schottky/Germanium diodes, ensuring accuracy at low power.

2. SWR Calculation

Standing Wave Ratio is derived from the Reflection Coefficient ($\Gamma$):

$$\Gamma = \frac{V_{ref} + V_{diode}}{V_{fwd} + V_{diode}}$$

$$SWR = \frac{1 + \Gamma}{1 - \Gamma}$$

The system samples 60 times per loop and holds the highest detected value for 3 seconds to capture SSB voice peaks.

⚙️ Calibration Procedure

- 50Ω Match: Connect a 50Ω dummy load; $V_{ref}$ should be $0.00\text{V}$, resulting in SWR 1.0.

- 100Ω Mismatch: Connect a 100Ω load. The meter must show SWR 2.00.

  - If SWR < 2.0: Increase diode_vf.

  - If SWR > 2.0: Decrease diode_vf.

- No Load (Open Circuit): Transmit briefly with no antenna. $V_{fwd}$ and $V_{ref}$ will equalize, triggering the HIGH SWR Red Alert.

📡 SWR in HAM Radio Communications

In HF (High Frequency) communications, SWR measures impedance matching between the radio and the antenna system. Most modern transceivers require a 50Ω environment. When impedance deviates, energy is reflected back to the source.

While an SWR of 1.5:1 is ideal, ratios above 3:1 are hazardous. High SWR causes excessive heat in the Final PA (Power Amplifier) transistors, potentially leading to catastrophic hardware failure. This digital monitor’s High SWR Alert and 3-second peak hold provide a critical safety margin by visualizing transients that traditional analog needles often miss during rapid voice transmissions.